# ELEE 2045 Spring 2026 Lab 2

GitHub Classrooms Link: https://classroom.github.com/a/4f_7Nqr-

### Objectives:

- Hardware Mastery: Demonstrate that you can design, load, and run programs on the M5StickCPlus.

- Serial Communication: Practice serial-based communication between your M5StickCPlus and a program running on your computer.

### Part 1: The Standalone Reaction Game

You are going to design a standalone reaction-time game. This requires you to manage "states" (e.g., waiting for the user, waiting for the stimulus, showing the result).

Functionality:

Note: The large front button labeled "M5" is Button A.

Your program must implement the following 5 states:

- Idle State:

  - The screen displays "Press M5 to Start".

- Wait State:

  - When Button A is pressed, the screen turns RED.

  - The device waits for a random amount of time (between 2 to 5 seconds).

- Reaction State:

  - After the random time elapses, the screen turns GREEN immediately.

- Result State:

  - The user has pressed Button A while in the Reaction state.

  - Calculate the time difference between the screen turning green and the button press.

  - Display the reaction time in milliseconds (e.g., "245 ms").

- Foul State:

  - If the user presses the button while still in the Wait state, display "Too Early!"

Important Note on Randomness:

Computers are deterministic. You must consider hardware random number generation to ensure your wait times are actually random. Refer to the following documentation:

- Arduino Reference: Random

- ESP-IDF Reference: Hardware Random

Creative Extensions (Required):

The functionality above is the bare minimum requirement. You must add two (2) distinct additional features to the reaction timer.

- Examples: A visualization to display the elapsed time, a different modality to handle the reaction (e.g., motion/IMU), displaying statistics over many runs, or something more creative.

### Part 2: The Digital Photo Frame

In this part, you will establish communication between the M5StickCPlus and your computer over the USB serial port. You will write a program for your computer that communicates with a program you write for your M5StickCPlus, implementing a digital photo frame.

#### 1. The Python Program

You must figure out a way to send an image from your computer to the M5StickCPlus over the serial port.

- Input: The program should read a folder of images (specified on the command line).

- Processing: Open the images and resize them appropriately. Note that the M5StickCPlus2 has a 135x240 display. You must handle arbitrarily sized images in the folder.

- Transmission: Send the images periodically (interval specified on the command line) to the M5StickCPlus over the serial port.

- Display Logic: It is up to you to decide how to handle the aspect ratio/cropping of the images.

#### 2. The M5StickCPlus Program

The microcontroller program should:

- Receive the images from the Serial port.

- Display the images on the screen.

- Toggle the screen to black (hiding the image) when Button A is pressed, and back to the image when pressed again.

Creative Extensions (Required):

As with Part 1, you must implement two (2) distinct additional features beyond the ones specified here.

- Examples:

  - Send captions alongside the images.

  - Implement a simple GIF player (watch memory limitations!).

  - Implement "plumbing" features like image compression or local storage (allowing the frame to run while disconnected).

  - Support stereo images (side-by-side or cross-eye viewing).

  - Support automatic rotation of the image based on the physical rotation of the M5StickCPlus (IMU).

The M5GFX library has numerous image functions to work with.  See the documentation for details.

### Submission Requirements

1. GitHub Repository:

Your repository must contain:

- Part 1 Folder: Source code for the reaction game.

- Part 2 Folder: Source code for the M5StickCPlus and the Python script.

- README.md: Must include your Name, Headshot, links to your Part 1 Video and Part 2 Video, and a brief description of your 4 Creative Extensions.  A template is provided with this assignment.

2. Video Demo:

- Part 1: Demonstrate the Idle, Wait, React, and Foul states, plus your extensions.

- Part 2: Show the Python script running (command line usage) and the screen updating. Demonstrate the toggle feature and your extensions.

### Grading Rubric (100 Points Total)

| Category | Points | Description |
| --- | --- | --- |
| Part 1: Base Functionality | 25 | Correctly implements all 5 states (Idle, Wait, Reaction, Result, Foul). Handles randomness correctly. |
| Part 1: Creative Extensions | 15 | Implements two distinct, working features beyond the base requirements (10 points for feature 1, 5 points for feature 2). |
| Part 2: Base Functionality | 25 | Python script correctly handles arguments, resizes, and sends images. M5Stick correctly receives/displays them. Toggle button works. |
| Part 2: Creative Extensions | 15 | Implements two distinct, working features beyond the base requirements (10 points for feature 1, 5 points for feature 2). |
| Code Quality & Git | 10 | Clean code (comments, variable names). Logic is non-blocking where appropriate. Commit history reflects progress. |
| Documentation & Video | 10 | README.md is complete. Videos clearly demonstrate all requirements and extensions. Videos are no more than 2 minutes each and include voiceover explaining how the programs work. |