ELEE 2045 Spring 2026 Assignment 3: Distributed MQTT Security System

GitHub Classrooms Link: https://classroom.github.com/a/zXUTTuDX

Objectives:

- IoT Protocol Integration: Understand and implement the MQTT (Message Queuing Telemetry Transport) protocol for lightweight, bidirectional communication.

- Distributed Systems: Design a system where an embedded C++ device (M5StickCPlus) and a desktop Python application work synchronously over WiFi

- State Management: Handle complex, multi-state logic (Armed, Disarmed, Grace Periods) across a network.

Overview

You will design and implement a distributed, mock security system. Your system will consist of a physical sensor/control panel running on the M5StickCPlus and a remote monitoring dashboard running on your computer. These two components must communicate exclusively over the internet (WiFi on the M5StickCPlus) using an MQTT broker.

You are given significant leeway in designing what physical property you are securing and how the user interface looks, provided all functional requirements are met.

Important note before you start working on this: If you are using the eduoracle.ugavel.com broker, please put all of your topics under elee2045sp26/yourid/whatevertopics, e.g., elee2056sp26/kjohnsen/movement

Part 1: The Security Sensor (Arduino / C++)

You will write a C++ program for your M5StickCPlus (1 or 2) that acts as the physical intrusion sensor and a localized control panel.

Functionality:

- Sensing Mechanism: You must design a way to detect a physical "breach." You can use the built-in 6-axis IMU (e.g., detecting if a door is opened or a box is moved), the microphone (e.g., a loud noise), or potentially external hardware (e.g., a proximity sensor or reed-switch).

- Local UI & Control: The M5Stick must display the current system state (e.g., color-coded screens, text). The user must be able to arm, disarm, and change the modes of the system directly using the physical buttons on the M5Stick.  The “code” for the system is up to you, but the minimum requirement would be simply pressing the right button.

- Alerting: The device must provide localized alerts (screen flashing, LED, or built-in buzzer) when an alarm is tripped.

Part 2: The Security Monitor (Python)

You will write a Python program for your computer that serves as a remote dashboard for the security system.

Functionality:

- Remote UI & Control: The Python program (whether a command-line interface or a graphical GUI) must display the current state of the system and allow the user to remotely arm, disarm, and change the system mode.

- Data Presentation: The monitor must display telemetry data (e.g., showing what tripped the sensor or current sensor readings).

- Connection Monitoring: The Python program must actively monitor the connection status of the M5Stick. If the M5Stick disconnects from the Wi-Fi or loses power, the Python monitor must detect this (e.g., using a “heartbeat”) and alert the user on the computer.

Required System Modes

Your system must support (at least) the following distinct modes. The active mode must be synchronized and adjustable from both the M5Stick and the Python Monitor:

- Off / Monitoring Mode: The system is disarmed. No alarms will sound if the sensor is triggered. However, the system must still actively report its sensor state to the Monitor (e.g., "Door Opened" but no alarm).

- Home Mode: The system is armed. If the sensor is triggered, the alarm state (on both the M5Stick and the Monitor) trips immediately without warning.

- Away Mode: The system is armed but accommodates a user entering the premises. If the sensor is triggered, the system enters a "grace period" (e.g., 10 seconds) where it warns the user locally on the M5Stick. If the system is not disarmed within this time, the full alarm state is triggered.

Creative Extensions (Required)

The functionality above is the bare minimum requirement. You must implement one additional feature to your overall system.

- Examples: * Maintain a local log file of all security events on the Python monitor with timestamps.

  - Integrate a PC webcam to take a photo when the alarm is tripped.

  - Send an email or SMS alert from the Python script when a breach occurs.

  - Support multiple M5Stick "zones" (e.g., Front Door, Back Door) communicating with the single Monitor.

  - A way to enter a more elaborate code (e.g., sequence of button presses, sounds, taps).

Submission Requirements

1. GitHub Repository: Your repository must contain:

- SecuritySensor Folder: Source code (C++/Arduino) for the M5StickCPlus.

- SecurityMonitor Folder: Source code (Python) for the computer.

- README.md: Must include your Name, Headshot, a link to your Video Demo, an explanation of your system architecture/MQTT topics, a user interface guide, and a brief description of your 2 Creative Extensions. A template is provided with this assignment.

2. Video Demo:

- Demonstrate all 3 modes (Off, Home, Away) being set from both devices.

- Show the physical sensor being triggered in both Home and Away modes.

- Demonstrate the Python monitor detecting when the M5Stick is powered off or disconnected.

- Showcase your creative extension.

- The video must be no more than 3 minutes and include a voiceover explaining how the programs work.

Rubric

| Category | Points | Description |
| --- | --- | --- |
| Security Sensor Base (C++) | 25 | Correctly implements hardware sensing, localized alerts, and allows mode control via physical buttons. Communicates successfully via MQTT. |
| Security Monitor Base (Python) | 20 | Correctly displays system state, allows remote mode control, displays telemetry data, and successfully monitors device connection status. |
| System Logic & Sync | 20 | The 3 required modes (Off, Home, Away with grace period) function exactly as specified. State changes sync quickly between both devices. |
| Creative Extension | 10 | Implements a distinct, working feature beyond the base requirements. |
| Code Quality & Git | 15 | Clean code (comments, variable names). Properly named programs and structure. Commit history reflects steady progress. |
| Documentation & Video | 10 | README.md is complete with architecture and UI explanations. Video clearly demonstrates all requirements, modes, and connection monitoring under 3 minutes with voiceover. |